## Technical Report: Our EV Energy Consumption Model (V2)

### 1. Problem Statement

Our goal is to accurately predict how much energy an electric vehicle (EV) uses during a trip. This is a major challenge because real-world driving involves a complex mix of physics and unpredictable factors like weather. We want to minimize these errors to provide drivers with a truly reliable range prediction tool.

### 2. What We Added (Comparison: V1 vs. V2)

To move from a basic theoretical model (V1) to a "Digital Twin" (V2), we enriched our dataset with several key elements:

- Real Efficiency Curves: Instead of using a fixed percentage, we integrated real-world data curves from Renault. These allow us to calculate efficiency based on motor speed ($\eta_{pt}$) and battery temperature ($\eta_{bat}$).

- Environmental Context: We added a rain_proxy to detect if the road is wet or dry and included the outside ambient temperature.

- Vehicle Load & State: We now include the payload_kg (weight of passengers/luggage) and the SOC_pct (current battery level) to make our physics calculations more realistic.

- Auxiliary Power: We integrated the real power used by the car's electronics (P_aux_load_kW) directly from OBD-II data.

### 3. Methodology: Our Data Process

We processed this data through a two-step hybrid "Grey-Box" approach:

#### A. The Physics Core (Deterministic Model)

We used the raw data to calculate the physical forces slowing the car down:

- Aerodynamics: We calculate the force of air against the car. We added a "corrector" (2.2) because real-world air is never perfectly still.

- Rolling & Gravity: We calculate tire friction based on road conditions (wet/dry) and the energy needed to climb hills using elevation changes.

- Inertia: We used a corrector (0.4) for acceleration to better match real-world friction and tire grip.

#### B. Machine Learning (The Calibration Layer)

Even with good physics, our initial model had a 9.83% error. To fix this, we used an XGBoost Regressor.

- What it does: Instead of predicting everything, the AI only predicts the mistake made by the physics model.

- The Logic: Our final result is the sum of the physics calculation and the AI's correction: $E_{hybrid} = E_{physics} + \text{AI Correction}$.

### 4. Results & Analysis

Comparing our model to real trip data showed a massive jump in precision:

| Metric | Physics Model (V1) | Our Hybrid Model (V2) |
| --- | --- | --- |
| Error (MAPE) | 9.83% | 0.91% |

Perfect Alignment: Our "Parity Plot" shows that our predictions now align almost perfectly with actual measurements.

Stability: The error stays near zero throughout the trip, meaning our model doesn't "drift" or get lost over long distances.

### 5. Points for Improvement

To make our Digital Twin even better, we have identified these next steps:

- Heating & Cooling: We plan to add a sub-model for the HVAC system (Heating/Air Conditioning), as it is a huge energy consumer depending on the weather.

- Battery Health (SOH): We want to use the small remaining errors to detect "Battery State-of-Health" degradation over time.

- Braking Regeneration: We currently estimate energy recovery at 70% during braking. We could refine this by using real-time motor torque data.

- Real-Time Integration: Our next goal is to optimize the code to run live inside the vehicle, updating the range every second based on current driving.